ECE 6213 – Design of VLSI Circuits

Fall 2022 – The George Washington University – Dr. Jerry Wu

Osama Yousuf – HW 2

Problem 1 – 35 points

- Obtain the equation and simplify it accordingly

The equation is: Odynamic=~A&B(C&~B)] &  B. My work is attached below:

The simplified expressions are: Ostatic=~A | ~B=~(AB), and Odynamic=~AB. My working is below:

- Discuss the possible issue with the original circuit

Let’s calculate the timing delays for the two outputs:

Temp0=2 nS

Temp2=0.8 nS

Temp1=0.8+2=2.8 nS

Ostatic=1.5+ 2.8 nS (longer delay out of its two inputs) =4.3 nS

Bdelay=0.8+0.8=1.6 nS

Odynamic=2+4.3=6.3 nS

The issue thus in the circuit is that even though we wanted the Bdelay to be the critical path for Odynamic, the gate delay for Ostatic is greater and thus instead the total delay is longer than intended.

Problem 2 – 55 points

- Create a truth table for “excess-5” code to BCD

| Excess-5 input |  |  |  | BCD output |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| w | x | y | z | a | b | c | d |
| 0 | 1 | 0 | 1 | 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |
| 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 0 | 1 | 1 | 0 |
| 1 | 1 | 0 | 0 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 0 | 1 | 0 | 0 | 1 |

Rest of the cases are don’t care X.

- Derive and optimize each output (a, b, c, d) as a function of the inputs (by K-maps based on your truth table)

K-Maps:

| Output a |  | y, z |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| w, x | 00 | X | X | X | X |
|  | 01 | X |  |  |  |
|  | 11 |  | 1 | X | 1 |
|  | 10 |  |  |  |  |

a(w, x, y, z) = wxz + wxy

Verilog: assign a = (w & x & z) | (w & x & y);

| Output b |  | y, z |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| w, x | 00 | X | X | X | X |
|  | 01 | X |  |  |  |
|  | 11 | 1 |  | X |  |
|  | 10 |  | 1 | 1 | 1 |

b(w, x, y, z) = x’y + x’z + xy’z’

Verilog: assign b = (~x & z) | (~x & y) | (x & ~y & ~z);

| Output c |  | y, z |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| w, x | 00 | X | X | X | X |
|  | 01 | X |  | 1 |  |
|  | 11 | 1 |  | X |  |
|  | 10 | 1 |  | 1 |  |

c(w, x, y, z)  = y’z’ + yz

Verilog: assign c = (y & z) | (~y & ~z);

| Output d |  | y, z |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 00 | 01 | 11 | 10 |
| w, x | 00 | X | X | X | X |
|  | 01 | X |  |  | 1 |
|  | 11 | 1 |  | X | 1 |
|  | 10 | 1 |  |  | 1 |

d(w, x, y, z)  = z’

Verilog: assign d = ~z;

- Model a converter circuit at gate level in structural style

Problem 3 – 10 points

- Following is incomplete Verilog code, please finish based on the synthesis result on the attached schematic.

F = A & B & AND_OR_Bits(E)

Y1 = F & C

Y2 = F | D