ECE 6213 – Design of VLSI Circuits

Fall 2022 – The George Washington University – Dr. Jerry Wu

Osama Yousuf – HW 5

Problem 3 – Finite State Machines

Mealy State Machine

Following is a list of key points for a Mealy  state machine.

- The output is a function of the current state as well as the input

- The input can directly cause a change in the output, which might violate time requirements.

- Compared to a Moore state machine, a Mealy state machine often has less total number of states.

Code: mealy.v

Testbench: mealy_tb.v

Testbench Features

- My testbench is self-reporting

- It starts with a reset to the WhiteState, and checks for the validity of every state transition sequentially.

- I implemented events for:

  - Resetting the state machine

  - Updating testbench cycles

  - Watchdog for exiting the simulation

  - Looping over the various states of the state machine and checking for validity

- I implemented logic for counting errors in every test case, as well as for counting total errors in the simulation

- I also implemented a simple task for printing the status of the errors encountered for self-reporting

The final text output of my testbench is given below; all test cases pass without any errors:

Waveform snippets for a few cases from my testbench are also displayed below:

- Firstly, the last 4 signals in my waveform indicate the state encodings for each of the four states. These are:

  - WhiteState → white_state → 00

  - RedState → red_state → 01

  - GreenState → green_state → 10

  - BlueState → blue_state → 11

- It can be seen that the asynchronous reset at the start means that both current and next states are white.

- When the red input is asserted, the next state becomes red, and the output of the FSM (mealy_out) is asserted, both at the exact same time. This is correct expected behavior since we have a mealy FSM; the input causes a direct change with the output.

- When the red input is made 0 again, the FSM correctly cycles back to the white state.

- A similar thing happens for other transitions, namely from white state to green state (and back), and white state to blue state (and back).

Moore State Machine

Following is a list of key points for a Moore state machine.

- The output is a function of only the current state.

- As a result, the output changes only after a state transition has occurred, contrary to a Mealy state machine.

- Compared to a Mealy state machine, a Moore state machine often has higher total number of states.

Code: moore.v

Testbench: moore_tb.v

Testbench Features

- My testbench is self-reporting

- It starts with a reset to the WhiteState, and checks for the validity of every state transition sequentially.

- I implemented events for:

  - Resetting the state machine

  - Updating testbench cycles

  - Watchdog for exiting the simulation

  - Looping over the various states of the state machine and checking for validity

- I implemented logic for counting errors in every test case, as well as for counting total errors in the simulation

- I also implemented a simple task for printing the status of the errors encountered for self-reporting

The final text output of my testbench is given below; all test cases pass without any errors:

Waveform snippets for a few cases from my testbench are also displayed below:

- Firstly, the last 4 signals in my waveform indicate the state encodings for each of the four states. These are:

  - WhiteState → white_state → 00

  - RedState → red_state → 01

  - GreenState → green_state → 10

  - BlueState → blue_state → 11

- It can be seen that the asynchronous reset at the start means that both current and next states are white.

- When the red input is asserted, the next state becomes red, but the output of the FSM (moore_out) is not asserted at the same clock. This is correct expected behavior since we have a Moore FSM; the input does not cause a direct change to the output. The output is asserted in the next clock cycle.

- When the red input is made 0, the FSM correctly cycles back to the white state.

- A similar thing happens for other transitions, namely from white state to green state (and back), and white state to blue state (and back).

Note: Instead of $fopen and $fclose, I used the “>” operator to directly re-direct terminal output to a text files for both of my FSMs (Mealy and Moore), as follows:

For Mealy output:

- iverilog -o mealy mealy.v mealy_tb.v

- vvp mealy > mealy_output.txt

For Moore output:

- iverilog -o moore moore.v moore_tb.v

- vvp moore > moore_output.txt

The end result are identical text files that contain all of the $display information from the two test benches, as shown in the snippets below:

The waveform files are also provided in the fsm folder (in .vcd format).